GEOTECHNICAL INVESTIGATION REPORT

FOR PROPOSED TRANSMISSION TOWER

TOWER SUITABILITY ANALYSIS

Project: Transmission line

Client: TAMS Tower Suitability

Consultant: Planeteye Infra AI

Project Location: Nirona, Bhuj, Gujarat

Coordinates: 23.446103°N, 69.599508°E

Purpose: Construction of Transmission Tower — Nirona, Bhuj screening

Report Classification: GIS BASED PRELIMINARY SCREENING

Report Date: 27 August 2026

Report ID: TAMS-GEO-NIRONA-SAMPLE

This document is a GIS-based preliminary geotechnical screening report. It is not a substitute for borehole investigation, laboratory testing, or structural foundation design unless field-validated data are explicitly identified herein as MEASURED.

# Table of Contents

PART A — Front Matter

1. Executive Summary

2. Scope and Objective of Investigation

3. Site and Geological Context

4. Subsurface Soil Profile

PART B — Transmission Line Report Body (Gujarat format)

1. Soil Test Summary

2. Engineering Analysis

2.1 Foundation Design Parameters

2.2 Adopted Soil Parameters

2.4 SBC Calculation — Detailed Sample

2.5 SBC Values for Location

2.6 SBC Variation with Depth

2.7 Settlement Check

2.8 Final Recommended SBC for Design

3. Pile Foundation Analysis and Design

3.1 Design Parameters

3.2 Adopted Soil Parameters (Layer-wise)

3.3 Cross-Sectional Area of Piles

3.4 Vertical Load Capacity (Detailed Sample)

3.5 Summary — Vertical Capacity

3.6 Uplift Capacity (Sample)

3.7 Uplift Load Capacity Summary

3.8 Lateral Load Capacity

3.9 Governing Design Condition

4. California Bearing Ratio (CBR)

5. Earth Resistivity

6. Foundation Recommendation

7. Investigation Limitations

8. References

ANNEXURE A — Depth-wise Soil Data

ANNEXURE B — Source Data and Provenance

ANNEXURE C — Engineering Calculation Sheets

# 1. Executive Summary

A preliminary geotechnical screening assessment has been prepared for the proposed transmission tower location at Nirona, Bhuj, Gujarat (23.446103°N, 69.599508°E). The nominal investigation depth considered for this screening is 2.0 m below ground level.

Available subsurface information is derived primarily from open GIS soil property models (ISRIC SoilGrids 2.0), aggregated into engineering depth intervals of 0.0–0.5 m, 0.5–1.0 m, 1.0–1.5 m and 1.5–2.0 m. Grain-size fractions, bulk density and related modelled properties are reported with status MODELLED. Laboratory classification parameters (Atterberg limits, SPT N-value, UCS, soaked CBR, field resistivity, groundwater table) are not available from remote sensing alone and are recorded as FIELD TEST REQUIRED or NO DATA.

Preliminary engineering calculations (safe bearing capacity and pile vertical/uplift capacities) are presented only where scientifically defensible input parameters exist. Where inputs are insufficient, calculation status is stated as INSUFFICIENT DATA. Settlement assessment requires tower foundation load and is currently recorded as TOWER_LOAD_REQUIRED.

Report classification for this location: GIS BASED PRELIMINARY SCREENING. This report must not be interpreted as a field-validated geotechnical investigation unless MEASURED borehole / laboratory data are present and explicitly identified.

Overall data readiness: 75.6% of tracked parameters available (68/90). Field validation required: Yes.

# 2. Scope and Objective of Investigation

## 2.1 Project Description

Project: Transmission line. Purpose: Construction of Transmission Tower — Nirona, Bhuj screening. Client: TAMS Tower Suitability. Consultant: Planeteye Infra AI.

## 2.2 Location Details

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| Location | Nirona, Bhuj, Gujarat |  |
| Latitude | 23.446103°N | 95 |
| Longitude | 69.599508°E |  |
| Elevation | 95 | MODELLED |
| Slope | 1.5 | DERIVED |
| Land cover hint | barren | MODELLED |

## 2.3 Objective of Investigation

The objective of this screening is to provide location-specific soil and foundation intelligence to support tower suitability decisions, identify data gaps requiring field/laboratory verification, and present preliminary bearing and pile capacity calculations where inputs permit.

## 2.4 Investigation Methodology

Methodology comprises: (a) point query of ISRIC SoilGrids 2.0 modelled soil properties; (b) thickness-weighted aggregation of source depth bands into 0.5 m engineering intervals to 2.0 m; (c) USDA texture derivation and preliminary material description; (d) optional engineering parameter estimation with published correlations where valid; (e) IS 6403-aligned SBC and IS 2911-aligned pile static calculations when c, φ and γ are available; (f) explicit documentation of missing field tests.

## 2.5 Data Sources and Limitations

Primary soil source: ISRIC SoilGrids 2.0 (~250 m resolution, modelled means — not borehole samples). Terrain: Open-Meteo elevation. Context distances and land cover may be drawn from live OSM/OSRM signals when available. SoilGrids source centimetre bands are retained only in Annexure B and are not presented as laboratory depth samples in the main body.

# 3. Site and Geological Context

## 3.1 Site Location

The site is located at Nirona, Bhuj, Gujarat. Coordinates used for all queries are 23.446103°N, 69.599508°E.

## 3.2 Surface Conditions

Land cover indication: barren (MODELLED). This indication is for screening context only and is not a cadastral or geotechnical surface classification.

## 3.3 GIS and Terrain Context

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| Elevation | 95 | MODELLED |
| Local slope (DEM) | 1.5 | DERIVED |
| Field investigation match | No | No field investigation within search radius |

# 4. Subsurface Soil Profile

## 4.1 Depth-wise Soil Profile

Generalised subsoil profile for screening purposes (engineering intervals 0.0–2.0 m):

| Layer | Depth Below G.L. (m) | Material Description | Data Status |
| --- | --- | --- | --- |
| Layer I | 0.0–0.5 | Clay loam — modelled texture estimate (Sand 38% · Silt 30% · Clay 22%) | MODELLED |
| Layer II | 0.5–1.0 | Sandy loam — modelled texture estimate (Sand 47% · Silt 19% · Clay 13%) | MODELLED |
| Layer III | 1.0–1.5 | Loamy sand — modelled texture estimate (Sand 52% · Silt 12% · Clay 6%) | MODELLED |
| Layer IV | 1.5–2.0 | Loamy sand — modelled texture estimate (Sand 52% · Silt 12% · Clay 6%) | MODELLED |

## 4.2 Soil Material Description

Material descriptions are preliminary modelled texture estimates derived from sand–silt–clay fractions. They are not laboratory-confirmed IS 1498 classifications unless MEASURED plasticity and grading data are available.

## 4.3 Soil Data Quality

Modelled coverage: 40%. Estimated coverage: 6.7%. Measured coverage: 0%. Missing critical parameters: SPT_N_VALUE, DIRECT_SHEAR_PARAMETERS, GROUNDWATER_LEVEL, FIELD_EARTH_RESISTIVITY, ATTERBERG_LIMITS, SOAKED_CBR, MATCHED_FIELD_INVESTIGATION.

# 1. SOIL TEST SUMMARY

Location —: Nirona, Bhuj, Gujarat

Coordinates: 23.446103°N  69.599508°E

Depth-wise soil properties for engineering intervals 0.0–2.0 m (metres only). Grain-size and density from SoilGrids are MODELLED numbers. Laboratory columns retain the Transmission-line table structure; missing lab tests are shown as FTR (FIELD TEST REQUIRED) — never fabricated as MEASURED.

### Table 1 — Depth intervals 0.5 m

| Depth (m) | Gravel % | Sand % | Silt % | Clay % | LL (%) | PL (%) | PI (%) | Soil Class | MDD (g/cc) | OMC (%) | Dry Density (g/cc) | FSI (%) | Bulk Density (g/cc) | UCS (kg/cm²) | SG | SBC (T/m²) | CBR (%) | Remarks | Ground Water Table (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.0–0.5 | FTR | 38 | 30 | 22.1 | FTR | FTR | FTR | Clay loam (USDA)** | FTR | FTR | 1.57 | FTR | 1.57 | FTR | FTR | 14.6 (CALC) | 2–7 (EST*) | Clay loam — modelled texture estimate (Sand 3… | NO DATA (GIS) |
| 0.5–1.0 | FTR | 47 | 19.4 | 13.2 | FTR | FTR | FTR | Sandy loam (USDA)** | FTR | FTR | 1.73 | FTR | 1.73 | FTR | FTR | 24.7 (CALC) | 8–18 (EST*) | Sandy loam — modelled texture estimate (Sand … | NO DATA (GIS) |
| 1.0–1.5 | FTR | 52 | 12 | 6 | FTR | FTR | FTR | Loamy sand (USDA)** | FTR | FTR | 1.85 | FTR | 1.85 | FTR | FTR | 36.2 (CALC) | 8–18 (EST*) | Loamy sand — modelled texture estimate (Sand … | NO DATA (GIS) |
| 1.5–2.0 | FTR | 52 | 12 | 6 | FTR | FTR | FTR | Loamy sand (USDA)** | FTR | FTR | 1.85 | FTR | 1.85 | FTR | FTR | 49 (CALC) | 8–18 (EST*) | Loamy sand — modelled texture estimate (Sand … | NO DATA (GIS) |

### Table 2 — Aggregated intervals 0–1 m and 1–2 m

| Depth (m) | Gravel % | Sand % | Silt % | Clay % | LL (%) | PL (%) | PI (%) | Soil Class | MDD (g/cc) | OMC (%) | Dry Density (g/cc) | FSI (%) | Bulk Density (g/cc) | UCS (kg/cm²) | SG | SBC (T/m²) | CBR (%) | Remarks | Ground Water Table (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0–1 | FTR | 42.5 | 24.7 | 17.6 | FTR | FTR | FTR | Sandy loam (USDA)** | FTR | FTR | 1.65 | FTR | 1.65 | FTR | FTR | 24.7 (CALC) | 2–7 (EST*) | Clay loam — modelled texture estimate (Sand 3… | NO DATA (GIS) |
| 1–2 | FTR | 52 | 12 | 6 | FTR | FTR | FTR | Loamy sand (USDA)** | FTR | FTR | 1.85 | FTR | 1.85 | FTR | FTR | 49 (CALC) | 8–18 (EST*) | Loamy sand — modelled texture estimate (Sand … | NO DATA (GIS) |

• FTR = FIELD TEST REQUIRED (laboratory / in-situ). Values are never fabricated as MEASURED.

• Sand / Silt / Clay / Dry Density / Bulk Density: MODELLED from ISRIC SoilGrids 2.0 (thickness-weighted).

• Soil Class marked (USDA)** is texture-triangle class — not IS 1498 without Atterberg limits.

• SBC/CBR marked (EST*) = texture screening estimate — not IS 6403 CALCULATED / not soaked CBR lab result.

• SBC marked (CALC) = IS 6403 net safe bearing capacity when c, φ, γ inputs are scientifically complete.

• Gravel % requires sieve analysis; SoilGrids cfvo is not reported as gravel %.

# 2. ENGINEERING ANALYSIS

## 2.1 Foundation Design Parameters

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| Foundation Type | 1.0 m × 1.0 m square isolated stub footing (screening assumption) | IS 6403 : 1981 |
| Depth of Foundation (Df) | 1.5 m | Site / screening condition |
| Allowable Settlement | 25 mm | IS 8009 (Part-1) |
| Factor of Safety (shear) | 2.5 | IS 6403 |
| Water Table | NO DATA (GIS) — not invented | Preliminary assessment |
| Width B / Length L | 1 m × 1 m | Screening default |

## 2.2 Adopted Soil Parameters (at 1.5 m depth — screening)

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| Unit weight γ | 18.1 | ESTIMATED |
| Friction angle φ | 31 | ESTIMATED |
| Cohesion c | FIELD TEST REQUIRED | FIELD TEST REQUIRED |
| Texture hint | Loamy sand | MODELLED / ESTIMATED |
| Code reference | IS 6403:1981 — Code of practice for determination of bearing capacity of shallow founda… |  |

Note: Adopted c, φ, γ for GIS screening are ESTIMATED or FIELD TEST REQUIRED. They are not borehole MEASURED values unless a matched field investigation is identified.

## 2.4 SBC CALCULATION — DETAILED SAMPLE

Preliminary IS 6403 net SBC computed at 4/4 depths using ESTIMATED φ and ESTIMATED c. Screening foundation 1×1 m, FoS=2.5.

Calculation status: CALCULATED.

### Calculation sheet — Df = 1.5 m

Step 1 — Input parameters: c, φ, γ, B, L, Df, FoS

Step 2 — Bearing capacity factors (IS 6403): Nq = e^(π tanφ)·tan²(45+φ/2); Nc = (Nq−1)·cotφ; Nγ = 2(Nq+1)tanφ  →  Nc=32.67, Nq=20.63, Nγ=25.99 —

Step 3 — Shape factors: sc=1+0.2(B/L); sq=1+0.2(B/L)tanφ; sγ=max(0.6, 1−0.4(B/L))  →  sc=1.200, sq=1.120, sγ=0.600 —

Step 4 — Depth factors: φ≥10°: dc=dq=1+0.2(Df/B)√tanφ; dγ=1  →  dc=1.233, dq=1.233, dγ=1.000 —

Step 5 — Surcharge (overburden): q = γ · Df  →  2.768 T/m²

Step 6 — Cohesion component: c · Nc · sc · dc  →  0 T/m²

Step 7 — Surcharge component: q · Nq · sq · dq  →  78.83 T/m²

Step 8 — Unit weight component: 0.5 · γ · B · Nγ · sγ · dγ  →  14.388 T/m²

Step 9 — Gross ultimate bearing capacity: qu = cohesion + surcharge + unit-weight components  →  93.218 T/m²

Step 10 — Net ultimate bearing capacity: qnu = qu − q  →  90.45 T/m²

Step 11 — Factor of safety: FoS (shear)  →  2.5 —

Step 12 — Net safe bearing capacity: qs = qnu / FoS  →  36.18 T/m²

Adopted preliminary net SBC: 36.2 (CALCULATED). Confidence: 35%.

## 2.5 SBC Values for Location

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| Location | Nirona, Bhuj, Gujarat | 23.446103°N 69.599508°E |
| φ (°) | 31 | ESTIMATED |
| c | FIELD TEST REQUIRED | FIELD TEST REQUIRED |
| γ | 18.1 | ESTIMATED |
| SBC (T/m²) | 36.2 | CALCULATED |

## 2.6 SBC Variation with Depth

| Depth (m) | Net Safe Bearing Capacity (T/m²) | Status |
| --- | --- | --- |
| 0.5 | 14.6 | CALCULATED |
| 1.0 | 24.7 | CALCULATED |
| 1.5 | 36.2 | CALCULATED |
| 2.0 | 49 | CALCULATED |

## 2.7 Settlement Check

Final settlement not computed — tower foundation load / contact pressure required from structural design

Calculation status: TOWER_LOAD_REQUIRED.

Required inputs: TOWER_FOUNDATION_LOAD, FOUNDATION_DIMENSIONS, SOIL_MODULUS_OR_EQUIVALENT, POISSON_RATIO, INFLUENCE_FACTOR.

Missing inputs: TOWER_FOUNDATION_LOAD, SOIL_MODULUS_OR_EQUIVALENT.

Final settlement value is not reported. Tower foundation load / contact pressure and/or soil modulus are required before a numerical settlement can be stated.

## 2.8 Final Recommended SBC for Design

Preliminary recommended net SBC for screening: 36.2 T/m² (status CALCULATED). Confirm with field shear tests before structural design.

# 3. PILE FOUNDATION ANALYSIS & DESIGN

## 3.1 Design Parameters

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| Pile Type | Cast-in-situ RCC pile (screening) | IS 2911 (Part 1) |
| Pile Diameters | 450 mm and 600 mm | TAMS tower screening |
| Pile Depths | 1.0 m, 1.5 m, 2.0 m |  |
| FoS Compression | 2.5 | IS 2911 practice |
| FoS Uplift | 3.0 | IS 2911 practice |
| Method | c–φ static method only; SPT N never fabricated | IS 2911 (Part 1/Sec 2):2010 — bored cast-in-situ piles (static screening form) |

SPT N-values are never fabricated. Lateral capacity requires tower shear/moment and pile-head fixity.

Preliminary pile vertical/uplift computed for 6/6 configurations via c–φ static method. Lateral remains blocked (tower load / fixity required). SPT N not used.

## 3.2 Adopted Soil Parameters (Layer-wise)

| Layer | Depth (m) | Soil Type | c | φ (°) | γ |
| --- | --- | --- | --- | --- | --- |
| Layer I | 0.0–0.5 | Clay loam | FIELD TEST REQUIRED | 31 | 18.1 |
| Layer II | 0.5–1.0 | Sandy loam | FIELD TEST REQUIRED | 31 | 18.1 |
| Layer III | 1.0–1.5 | Loamy sand | FIELD TEST REQUIRED | 31 | 18.1 |
| Layer IV | 1.5–2.0 | Loamy sand | FIELD TEST REQUIRED | 31 | 18.1 |

## 3.3 Cross-Sectional Area of Piles

450 mm — Ap = πD²/4                 =  0.159 m²

600 mm — Ap = πD²/4                 =  0.2827 m²

## 3.4 VERTICAL LOAD CAPACITY (Detailed Sample — 450 mm × 2.0 m)

Diameter D                          =  0.450 m

Length L                            =  2.000 m

Calculation status                  =  PARTIAL

Area Ap                             =  0.159 m²

Perimeter                           =  1.4137 m

φ                                   =  31°

c                                   =  0 T/m²

γ                                   =  1.845 T/m³

Step 1 — Pile geometry: Ap = π D²/4; perimeter = π D

Step 2 — Effective overburden at tip: PD = γ · L → 3.69 T/m²

Step 3 — Bearing capacity factor Nq: Nq = e^(π tanφ)·tan²(45+φ/2) → 20.631 —

Step 4 — End bearing Qb: Qb = Ap·PD·Nq (bored; Nγ neglected) → 12.108 T

Step 5 — Earth pressure & wall friction: Ki = 1−sinφ; δ = 0.75φ

Step 6 — Shaft friction Qs: Qs = Σ (π D Δz · PD_mid · Ki · tanδ) [+ α c As if c>0] → 1.087 T

Step 7 — Ultimate vertical capacity: Qu = Qb + Qs → 13.194 T

Step 8 — Safe vertical capacity: Qsafe = Qu / 2.5 → 5.28 T

Step 9 — Uplift — self-weight: W = Ap · L · γ_concrete → 0.795 T

Step 10 — Safe uplift capacity: Qsafe_up = (W + Qs) / 3 → 0.63 T

Step 11 — Lateral capacity: requires tower shear/moment, head fixity, soil stiffness

## 3.5 Summary — Vertical Capacity

| Diameter | Depth | Safe Vertical (T) | Status |
| --- | --- | --- | --- |
| 450 mm | 1.0 m | 2.5 | CALCULATED |
| 450 mm | 1.5 m | 3.9 | CALCULATED |
| 450 mm | 2.0 m | 5.3 | CALCULATED |
| 600 mm | 1.0 m | 4.4 | CALCULATED |
| 600 mm | 1.5 m | 6.8 | CALCULATED |
| 600 mm | 2.0 m | 9.2 | CALCULATED |

## 3.6 UPLIFT CAPACITY (Sample — 450 mm × 2.0 m)

Safe uplift: 0.6 (CALCULATED). When CALCULATED, uplift uses shaft resistance / FoS uplift per IS 2911 screening form.

## 3.7 UPLIFT LOAD CAPACITY SUMMARY

| Diameter | Depth | Safe Uplift (T) | Status |
| --- | --- | --- | --- |
| 450 mm | 1.0 m | 0.2 | CALCULATED |
| 450 mm | 1.5 m | 0.4 | CALCULATED |
| 450 mm | 2.0 m | 0.6 | CALCULATED |
| 600 mm | 1.0 m | 0.4 | CALCULATED |
| 600 mm | 1.5 m | 0.6 | CALCULATED |
| 600 mm | 2.0 m | 1 | CALCULATED |

## 3.8 LATERAL LOAD CAPACITY

| Diameter | Depth | Lateral Capacity (T) | Status |
| --- | --- | --- | --- |
| 450 mm | 1.0 m | FIELD TEST REQUIRED | FIELD TEST REQUIRED |
| 450 mm | 1.5 m | FIELD TEST REQUIRED | FIELD TEST REQUIRED |
| 450 mm | 2.0 m | FIELD TEST REQUIRED | FIELD TEST REQUIRED |
| 600 mm | 1.0 m | FIELD TEST REQUIRED | FIELD TEST REQUIRED |
| 600 mm | 1.5 m | FIELD TEST REQUIRED | FIELD TEST REQUIRED |
| 600 mm | 2.0 m | FIELD TEST REQUIRED | FIELD TEST REQUIRED |

## 3.9 Governing Design Condition for Transmission Tower Foundation

Transmission line tower foundations are generally governed by vertical compression, uplift due to wind/conductor imbalance, and lateral wind loads (IS 875 Part 3). GIS screening does not replace structural load combinations or field pile design.

## 3.10 Detailed Configuration Sheets (all diameters × depths)

### Pile Foundation — 450 mm Diameter

### 450 mm dia × 1.0 m depth

Diameter D                          =  0.450 m

Length L                            =  1.000 m

Calculation status                  =  PARTIAL

Area Ap                             =  0.159 m²

Perimeter                           =  1.4137 m

φ                                   =  31°

c                                   =  0 T/m²

γ                                   =  1.845 T/m³

A. Vertical Capacity / B. Uplift Capacity — calculation steps:

Step 1 — Pile geometry: Ap = π D²/4; perimeter = π D

Step 2 — Effective overburden at tip: PD = γ · L → 1.845 T/m²

Step 3 — Bearing capacity factor Nq: Nq = e^(π tanφ)·tan²(45+φ/2) → 20.631 —

Step 4 — End bearing Qb: Qb = Ap·PD·Nq (bored; Nγ neglected) → 6.054 T

Step 5 — Earth pressure & wall friction: Ki = 1−sinφ; δ = 0.75φ

Step 6 — Shaft friction Qs: Qs = Σ (π D Δz · PD_mid · Ki · tanδ) [+ α c As if c>0] → 0.272 T

Step 7 — Ultimate vertical capacity: Qu = Qb + Qs → 6.326 T

Step 8 — Safe vertical capacity: Qsafe = Qu / 2.5 → 2.53 T

Step 9 — Uplift — self-weight: W = Ap · L · γ_concrete → 0.398 T

Step 10 — Safe uplift capacity: Qsafe_up = (W + Qs) / 3 → 0.22 T

Step 11 — Lateral capacity: requires tower shear/moment, head fixity, soil stiffness

| Capacity | Result | Status |
| --- | --- | --- |
| Safe vertical | 2.5 | CALCULATED |
| Safe uplift | 0.2 | CALCULATED |
| Lateral | FIELD TEST REQUIRED | FIELD TEST REQUIRED |

### 450 mm dia × 1.5 m depth

Diameter D                          =  0.450 m

Length L                            =  1.500 m

Calculation status                  =  PARTIAL

Area Ap                             =  0.159 m²

Perimeter                           =  1.4137 m

φ                                   =  31°

c                                   =  0 T/m²

γ                                   =  1.845 T/m³

A. Vertical Capacity / B. Uplift Capacity — calculation steps:

Step 1 — Pile geometry: Ap = π D²/4; perimeter = π D

Step 2 — Effective overburden at tip: PD = γ · L → 2.768 T/m²

Step 3 — Bearing capacity factor Nq: Nq = e^(π tanφ)·tan²(45+φ/2) → 20.631 —

Step 4 — End bearing Qb: Qb = Ap·PD·Nq (bored; Nγ neglected) → 9.081 T

Step 5 — Earth pressure & wall friction: Ki = 1−sinφ; δ = 0.75φ

Step 6 — Shaft friction Qs: Qs = Σ (π D Δz · PD_mid · Ki · tanδ) [+ α c As if c>0] → 0.611 T

Step 7 — Ultimate vertical capacity: Qu = Qb + Qs → 9.692 T

Step 8 — Safe vertical capacity: Qsafe = Qu / 2.5 → 3.88 T

Step 9 — Uplift — self-weight: W = Ap · L · γ_concrete → 0.596 T

Step 10 — Safe uplift capacity: Qsafe_up = (W + Qs) / 3 → 0.4 T

Step 11 — Lateral capacity: requires tower shear/moment, head fixity, soil stiffness

| Capacity | Result | Status |
| --- | --- | --- |
| Safe vertical | 3.9 | CALCULATED |
| Safe uplift | 0.4 | CALCULATED |
| Lateral | FIELD TEST REQUIRED | FIELD TEST REQUIRED |

### 450 mm dia × 2.0 m depth

Diameter D                          =  0.450 m

Length L                            =  2.000 m

Calculation status                  =  PARTIAL

Area Ap                             =  0.159 m²

Perimeter                           =  1.4137 m

φ                                   =  31°

c                                   =  0 T/m²

γ                                   =  1.845 T/m³

A. Vertical Capacity / B. Uplift Capacity — calculation steps:

Step 1 — Pile geometry: Ap = π D²/4; perimeter = π D

Step 2 — Effective overburden at tip: PD = γ · L → 3.69 T/m²

Step 3 — Bearing capacity factor Nq: Nq = e^(π tanφ)·tan²(45+φ/2) → 20.631 —

Step 4 — End bearing Qb: Qb = Ap·PD·Nq (bored; Nγ neglected) → 12.108 T

Step 5 — Earth pressure & wall friction: Ki = 1−sinφ; δ = 0.75φ

Step 6 — Shaft friction Qs: Qs = Σ (π D Δz · PD_mid · Ki · tanδ) [+ α c As if c>0] → 1.087 T

Step 7 — Ultimate vertical capacity: Qu = Qb + Qs → 13.194 T

Step 8 — Safe vertical capacity: Qsafe = Qu / 2.5 → 5.28 T

Step 9 — Uplift — self-weight: W = Ap · L · γ_concrete → 0.795 T

Step 10 — Safe uplift capacity: Qsafe_up = (W + Qs) / 3 → 0.63 T

Step 11 — Lateral capacity: requires tower shear/moment, head fixity, soil stiffness

| Capacity | Result | Status |
| --- | --- | --- |
| Safe vertical | 5.3 | CALCULATED |
| Safe uplift | 0.6 | CALCULATED |
| Lateral | FIELD TEST REQUIRED | FIELD TEST REQUIRED |

### Pile Foundation — 600 mm Diameter

### 600 mm dia × 1.0 m depth

Diameter D                          =  0.600 m

Length L                            =  1.000 m

Calculation status                  =  PARTIAL

Area Ap                             =  0.2827 m²

Perimeter                           =  1.885 m

φ                                   =  31°

c                                   =  0 T/m²

γ                                   =  1.845 T/m³

A. Vertical Capacity / B. Uplift Capacity — calculation steps:

Step 1 — Pile geometry: Ap = π D²/4; perimeter = π D

Step 2 — Effective overburden at tip: PD = γ · L → 1.845 T/m²

Step 3 — Bearing capacity factor Nq: Nq = e^(π tanφ)·tan²(45+φ/2) → 20.631 —

Step 4 — End bearing Qb: Qb = Ap·PD·Nq (bored; Nγ neglected) → 10.762 T

Step 5 — Earth pressure & wall friction: Ki = 1−sinφ; δ = 0.75φ

Step 6 — Shaft friction Qs: Qs = Σ (π D Δz · PD_mid · Ki · tanδ) [+ α c As if c>0] → 0.362 T

Step 7 — Ultimate vertical capacity: Qu = Qb + Qs → 11.125 T

Step 8 — Safe vertical capacity: Qsafe = Qu / 2.5 → 4.45 T

Step 9 — Uplift — self-weight: W = Ap · L · γ_concrete → 0.707 T

Step 10 — Safe uplift capacity: Qsafe_up = (W + Qs) / 3 → 0.36 T

Step 11 — Lateral capacity: requires tower shear/moment, head fixity, soil stiffness

| Capacity | Result | Status |
| --- | --- | --- |
| Safe vertical | 4.4 | CALCULATED |
| Safe uplift | 0.4 | CALCULATED |
| Lateral | FIELD TEST REQUIRED | FIELD TEST REQUIRED |

### 600 mm dia × 1.5 m depth

Diameter D                          =  0.600 m

Length L                            =  1.500 m

Calculation status                  =  PARTIAL

Area Ap                             =  0.2827 m²

Perimeter                           =  1.885 m

φ                                   =  31°

c                                   =  0 T/m²

γ                                   =  1.845 T/m³

A. Vertical Capacity / B. Uplift Capacity — calculation steps:

Step 1 — Pile geometry: Ap = π D²/4; perimeter = π D

Step 2 — Effective overburden at tip: PD = γ · L → 2.768 T/m²

Step 3 — Bearing capacity factor Nq: Nq = e^(π tanφ)·tan²(45+φ/2) → 20.631 —

Step 4 — End bearing Qb: Qb = Ap·PD·Nq (bored; Nγ neglected) → 16.143 T

Step 5 — Earth pressure & wall friction: Ki = 1−sinφ; δ = 0.75φ

Step 6 — Shaft friction Qs: Qs = Σ (π D Δz · PD_mid · Ki · tanδ) [+ α c As if c>0] → 0.815 T

Step 7 — Ultimate vertical capacity: Qu = Qb + Qs → 16.959 T

Step 8 — Safe vertical capacity: Qsafe = Qu / 2.5 → 6.78 T

Step 9 — Uplift — self-weight: W = Ap · L · γ_concrete → 1.06 T

Step 10 — Safe uplift capacity: Qsafe_up = (W + Qs) / 3 → 0.63 T

Step 11 — Lateral capacity: requires tower shear/moment, head fixity, soil stiffness

| Capacity | Result | Status |
| --- | --- | --- |
| Safe vertical | 6.8 | CALCULATED |
| Safe uplift | 0.6 | CALCULATED |
| Lateral | FIELD TEST REQUIRED | FIELD TEST REQUIRED |

### 600 mm dia × 2.0 m depth

Diameter D                          =  0.600 m

Length L                            =  2.000 m

Calculation status                  =  PARTIAL

Area Ap                             =  0.2827 m²

Perimeter                           =  1.885 m

φ                                   =  31°

c                                   =  0 T/m²

γ                                   =  1.845 T/m³

A. Vertical Capacity / B. Uplift Capacity — calculation steps:

Step 1 — Pile geometry: Ap = π D²/4; perimeter = π D

Step 2 — Effective overburden at tip: PD = γ · L → 3.69 T/m²

Step 3 — Bearing capacity factor Nq: Nq = e^(π tanφ)·tan²(45+φ/2) → 20.631 —

Step 4 — End bearing Qb: Qb = Ap·PD·Nq (bored; Nγ neglected) → 21.525 T

Step 5 — Earth pressure & wall friction: Ki = 1−sinφ; δ = 0.75φ

Step 6 — Shaft friction Qs: Qs = Σ (π D Δz · PD_mid · Ki · tanδ) [+ α c As if c>0] → 1.449 T

Step 7 — Ultimate vertical capacity: Qu = Qb + Qs → 22.974 T

Step 8 — Safe vertical capacity: Qsafe = Qu / 2.5 → 9.19 T

Step 9 — Uplift — self-weight: W = Ap · L · γ_concrete → 1.414 T

Step 10 — Safe uplift capacity: Qsafe_up = (W + Qs) / 3 → 0.95 T

Step 11 — Lateral capacity: requires tower shear/moment, head fixity, soil stiffness

| Capacity | Result | Status |
| --- | --- | --- |
| Safe vertical | 9.2 | CALCULATED |
| Safe uplift | 1 | CALCULATED |
| Lateral | FIELD TEST REQUIRED | FIELD TEST REQUIRED |

# 4. CALIFORNIA BEARING RATIO (CBR)

Measured (soaked) CBR and estimated CBR are reported separately. Estimated values must not be labelled as laboratory soaked CBR.

| Depth interval | Measured CBR (%) | Estimated CBR (%) | Measured status | Estimated status |
| --- | --- | --- | --- | --- |
| 0.0–0.5 m | NO DATA | 2–7 | NO DATA | ESTIMATED |
| 0.5–1.0 m | NO DATA | 8–18 | NO DATA | ESTIMATED |
| 1.0–1.5 m | NO DATA | 8–18 | NO DATA | ESTIMATED |
| 1.5–2.0 m | NO DATA | 8–18 | NO DATA | ESTIMATED |

# 5. EARTH RESISTIVITY

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| Measured earth resistivity | NO DATA | NO DATA |
| Estimated soil resistivity | NO DATA | NO DATA |

Estimated resistivity is never presented as a Wenner field measurement. Where no defensible correlation inputs exist, status remains NO DATA.

# 6. Foundation Recommendation

Based on the available gis based preliminary screening, the following preliminary observations are recorded for engineering guidance only:

1. Subsurface screening to 2.0 m indicates modelled soil conditions as summarised in Sections 4–5. Laboratory confirmation is required before final foundation design.

2. Net safe bearing capacity: preliminary 36.2 (CALCULATED).

3. Pile foundations (450 mm / 600 mm): vertical and uplift capacities are reported only where c–φ static inputs exist; otherwise FIELD TEST REQUIRED / INSUFFICIENT DATA. Lateral capacity remains blocked pending structural loads.

4. Recommended next field works: SPT or shear testing, Atterberg limits, soaked CBR, groundwater observation, and Wenner resistivity as listed in Section 7.

These recommendations do not constitute a sealed structural design certificate.

# 7. Investigation Limitations

1. PRELIMINARY GIS-BASED GEOTECHNICAL SCREENING — not a substitute for borehole investigation.

2. SoilGrids values are modelled at ~250 m resolution; report intervals are aggregated from source depth bands.

3. bdod is bulk density (g/cm³) and must never be interpreted as soil depth.

4. NO_DATA parameters are null — never replaced with 0.

5. SPT N, Atterberg limits, soaked CBR, groundwater depth, and field resistivity require physical testing.

6. Preliminary φ (if present) is ESTIMATED from texture mid-ranges; cohesion is FIELD_TEST_REQUIRED except drained sand c′≈0 ESTIMATED.

7. SBC: CALCULATED — Preliminary IS 6403 net SBC computed at 4/4 depths using ESTIMATED φ and ESTIMATED c. Screening foundation 1×1 m, FoS=2.5.

8. Settlement: TOWER_LOAD_REQUIRED — Final settlement not computed — tower foundation load / contact pressure required from structural design

9. Pile: Preliminary pile vertical/uplift computed for 6/6 configurations via c–φ static method. Lateral remains blocked (tower load / fixity required). SPT N not used.

10. SoilGrids texture class (0–30 cm screening): Clay loam

11. No field investigation within search radius

Missing critical data: SPT_N_VALUE; DIRECT_SHEAR_PARAMETERS; GROUNDWATER_LEVEL; FIELD_EARTH_RESISTIVITY; ATTERBERG_LIMITS; SOAKED_CBR.

# 8. References

1. IS 6403:1981 — Code of practice for determination of bearing capacity of shallow foundations.

2. IS 2911 (Part 1/Sec 2):2010 — Design and construction of pile foundations — Bored cast-in-situ piles.

3. IS 1498 — Classification and identification of soils for general engineering purposes.

4. IS 2720 (relevant parts) — Methods of test for soils.

5. IS 3043 — Code of practice for earthing.

6. ISRIC SoilGrids 2.0 — Global gridded soil information (modelled properties).

7. TAMS Tower Suitability — Geotechnical Intelligence module (GEO-1) calculation sheets.

# ANNEXURE A — Depth-wise Soil Data

Engineering depth intervals used in the main report (metres only). Sand / silt / clay / density values below are MODELLED SoilGrids means — not laboratory sieve or Proctor results.

| Report depth | Sand % | Silt % | Clay % | Bulk dens. | Dry dens. | pH | Org. C | USDA | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.0–0.5 m | 38 | 30 | 22.1 | 1.57 | 1.57 | 7.6 | 4.55 | Clay loam | MODELLED |
| 0.5–1.0 m | 47 | 19.4 | 13.2 | 1.73 | 1.73 | 7.42 | 2.2 | Sandy loam | MODELLED |
| 1.0–1.5 m | 52 | 12 | 6 | 1.85 | 1.85 | 7.3 | 1.2 | Loamy sand | MODELLED |
| 1.5–2.0 m | 52 | 12 | 6 | 1.85 | 1.85 | 7.3 | 1.2 | Loamy sand | MODELLED |

# ANNEXURE B — Source Data and Provenance

The following centimetre-scale intervals are SoilGrids source dataset bands. They are not laboratory sample depths and must not be confused with the engineering metre intervals in the main report.

| Source depth | From (m) | To (m) | Sand % | Silt % | Clay % | bdod (g/cm³) | pH | cfvo % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0-5cm | 0.00 | 0.05 | 30 | 38 | 28 | 1.45 | 7.8 | 4 |
| 5-15cm | 0.05 | 0.15 | 32 | 36 | 26 | 1.5 | 7.7 | 6 |
| 15-30cm | 0.15 | 0.30 | 38 | 30 | 23 | 1.55 | 7.6 | 10 |
| 30-60cm | 0.30 | 0.60 | 43 | 25 | 18 | 1.65 | 7.5 | 15 |
| 60-100cm | 0.60 | 1.00 | 48 | 18 | 12 | 1.75 | 7.4 | 22 |
| 100-200cm | 1.00 | 2.00 | 52 | 12 | 6 | 1.85 | 7.3 | 28 |

Aggregation method for report intervals: thickness-weighted mean of overlapping SoilGrids source layers.

Field investigation match: No field investigation within search radius

# ANNEXURE C — Engineering Calculation Sheets

This annexure consolidates calculation status for SBC, settlement and piles. Detailed step lists appear in Sections 6–7 when CALCULATED or PARTIAL.

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| SBC status | CALCULATED | IS 6403:1981 — Code of practice for determination of bearing capacity of shallow founda… |
| Settlement status | TOWER_LOAD_REQUIRED | Elastic settlement screening (form consistent with IS 8009 elastic approach) — prelimin… |
| Pile 450 mm × 2.0 m | PARTIAL | 5.3 |
| Pile 600 mm × 2.0 m | PARTIAL | 9.2 |

— End of Report —