GEOTECHNICAL INVESTIGATION REPORT

FOR PROPOSED TRANSMISSION TOWER

TOWER SUITABILITY ANALYSIS

Project: Transmission line

Client: TAMS Tower Suitability

Consultant: Planeteye Infra AI

Project Location: Patna, Bihar

Coordinates: 25.594100°N, 85.137600°E

Purpose: Construction of Transmission Tower — Patna sample screening

Report Classification: GIS BASED PRELIMINARY SCREENING

Report Date: 27 August 2026

Report ID: TAMS-GEO-PATNA-SAMPLE

This document is a GIS-based preliminary geotechnical screening report. It is not a substitute for borehole investigation, laboratory testing, or structural foundation design unless field-validated data are explicitly identified herein as MEASURED.

# Table of Contents

PART A — Front Matter

1. Executive Summary

2. Scope and Objective of Investigation

3. Site and Geological Context

4. Subsurface Soil Profile

PART B — Transmission Line Report Body (Gujarat format)

1. Soil Test Summary

2. Engineering Analysis

2.1 Foundation Design Parameters

2.2 Adopted Soil Parameters

2.4 SBC Calculation — Detailed Sample

2.5 SBC Values for Location

2.6 SBC Variation with Depth

2.7 Settlement Check

2.8 Final Recommended SBC for Design

3. Pile Foundation Analysis and Design

3.1 Design Parameters

3.2 Adopted Soil Parameters (Layer-wise)

3.3 Cross-Sectional Area of Piles

3.4 Vertical Load Capacity (Detailed Sample)

3.5 Summary — Vertical Capacity

3.6 Uplift Capacity (Sample)

3.7 Uplift Load Capacity Summary

3.8 Lateral Load Capacity

3.9 Governing Design Condition

4. California Bearing Ratio (CBR)

5. Earth Resistivity

6. Foundation Recommendation

7. Investigation Limitations

8. References

ANNEXURE A — Depth-wise Soil Data

ANNEXURE B — Source Data and Provenance

ANNEXURE C — Engineering Calculation Sheets

# 1. Executive Summary

A preliminary geotechnical screening assessment has been prepared for the proposed transmission tower location at Patna, Bihar (25.594100°N, 85.137600°E). The nominal investigation depth considered for this screening is 2.0 m below ground level.

Available subsurface information is derived primarily from open GIS soil property models (ISRIC SoilGrids 2.0), aggregated into engineering depth intervals of 0.0–0.5 m, 0.5–1.0 m, 1.0–1.5 m and 1.5–2.0 m. Grain-size fractions, bulk density and related modelled properties are reported with status MODELLED. Laboratory classification parameters (Atterberg limits, SPT N-value, UCS, soaked CBR, field resistivity, groundwater table) are not available from remote sensing alone and are recorded as FIELD TEST REQUIRED or NO DATA.

Preliminary engineering calculations (safe bearing capacity and pile vertical/uplift capacities) are presented only where scientifically defensible input parameters exist. Where inputs are insufficient, calculation status is stated as INSUFFICIENT DATA. Settlement assessment requires tower foundation load and is currently recorded as TOWER_LOAD_REQUIRED.

Report classification for this location: GIS BASED PRELIMINARY SCREENING. This report must not be interpreted as a field-validated geotechnical investigation unless MEASURED borehole / laboratory data are present and explicitly identified.

Overall data readiness: 56.7% of tracked parameters available (51/90). Field validation required: Yes.

# 2. Scope and Objective of Investigation

## 2.1 Project Description

Project: Transmission line. Purpose: Construction of Transmission Tower — Patna sample screening. Client: TAMS Tower Suitability. Consultant: Planeteye Infra AI.

## 2.2 Location Details

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| Location | Patna, Bihar |  |
| Latitude | 25.594100°N | 53 |
| Longitude | 85.137600°E |  |
| Elevation | 53 | MODELLED |
| Slope | 1.2 | DERIVED |
| Land cover hint | built | MODELLED |

## 2.3 Objective of Investigation

The objective of this screening is to provide location-specific soil and foundation intelligence to support tower suitability decisions, identify data gaps requiring field/laboratory verification, and present preliminary bearing and pile capacity calculations where inputs permit.

## 2.4 Investigation Methodology

Methodology comprises: (a) point query of ISRIC SoilGrids 2.0 modelled soil properties; (b) thickness-weighted aggregation of source depth bands into 0.5 m engineering intervals to 2.0 m; (c) USDA texture derivation and preliminary material description; (d) optional engineering parameter estimation with published correlations where valid; (e) IS 6403-aligned SBC and IS 2911-aligned pile static calculations when c, φ and γ are available; (f) explicit documentation of missing field tests.

## 2.5 Data Sources and Limitations

Primary soil source: ISRIC SoilGrids 2.0 (~250 m resolution, modelled means — not borehole samples). Terrain: Open-Meteo elevation. Context distances and land cover may be drawn from live OSM/OSRM signals when available. SoilGrids source centimetre bands are retained only in Annexure B and are not presented as laboratory depth samples in the main body.

# 3. Site and Geological Context

## 3.1 Site Location

The site is located at Patna, Bihar. Coordinates used for all queries are 25.594100°N, 85.137600°E.

## 3.2 Surface Conditions

Land cover indication: built (MODELLED). This indication is for screening context only and is not a cadastral or geotechnical surface classification.

## 3.3 GIS and Terrain Context

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| Elevation | 53 | MODELLED |
| Local slope (DEM) | 1.2 | DERIVED |
| Field investigation match | No | No field investigation within search radius |

# 4. Subsurface Soil Profile

## 4.1 Depth-wise Soil Profile

Generalised subsoil profile for screening purposes (engineering intervals 0.0–2.0 m):

| Layer | Depth Below G.L. (m) | Material Description | Data Status |
| --- | --- | --- | --- |
| Layer I | 0.0–0.5 | Clay loam — modelled texture estimate (Sand 35% · Silt 31% · Clay 35%) | MODELLED |
| Layer II | 0.5–1.0 | Clay loam — modelled texture estimate (Sand 39% · Silt 30% · Clay 31%) | MODELLED |
| Layer III | 1.0–1.5 | Clay loam — modelled texture estimate (Sand 42% · Silt 30% · Clay 28%) | MODELLED |
| Layer IV | 1.5–2.0 | Clay loam — modelled texture estimate (Sand 42% · Silt 30% · Clay 28%) | MODELLED |

## 4.2 Soil Material Description

Material descriptions are preliminary modelled texture estimates derived from sand–silt–clay fractions. They are not laboratory-confirmed IS 1498 classifications unless MEASURED plasticity and grading data are available.

## 4.3 Soil Data Quality

Modelled coverage: 40%. Estimated coverage: 6.7%. Measured coverage: 0%. Missing critical parameters: SPT_N_VALUE, DIRECT_SHEAR_PARAMETERS, GROUNDWATER_LEVEL, FIELD_EARTH_RESISTIVITY, ATTERBERG_LIMITS, SOAKED_CBR, MATCHED_FIELD_INVESTIGATION.

# 1. SOIL TEST SUMMARY

Location —: Patna, Bihar

Coordinates: 25.594100°N  85.137600°E

Depth-wise soil properties for engineering intervals 0.0–2.0 m (metres only). Grain-size and density from SoilGrids are MODELLED numbers. Laboratory columns retain the Transmission-line table structure; missing lab tests are shown as FTR (FIELD TEST REQUIRED) — never fabricated as MEASURED.

### Table 1 — Depth intervals 0.5 m

| Depth (m) | Gravel % | Sand % | Silt % | Clay % | LL (%) | PL (%) | PI (%) | Soil Class | MDD (g/cc) | OMC (%) | Dry Density (g/cc) | FSI (%) | Bulk Density (g/cc) | UCS (kg/cm²) | SG | SBC (T/m²) | CBR (%) | Remarks | Ground Water Table (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.0–0.5 | FTR | 34.6 | 30.6 | 34.8 | FTR | FTR | FTR | Clay loam (USDA)** | FTR | FTR | 1.45 | FTR | 1.45 | FTR | FTR | 10 (EST*) | 2–7 (EST*) | Clay loam — modelled texture estimate (Sand 3… | NO DATA (GIS) |
| 0.5–1.0 | FTR | 39.2 | 29.8 | 31 | FTR | FTR | FTR | Clay loam (USDA)** | FTR | FTR | 1.54 | FTR | 1.54 | FTR | FTR | 10 (EST*) | 2–7 (EST*) | Clay loam — modelled texture estimate (Sand 3… | NO DATA (GIS) |
| 1.0–1.5 | FTR | 42 | 30 | 28 | FTR | FTR | FTR | Clay loam (USDA)** | FTR | FTR | 1.6 | FTR | 1.6 | FTR | FTR | 10 (EST*) | 2–7 (EST*) | Clay loam — modelled texture estimate (Sand 4… | NO DATA (GIS) |
| 1.5–2.0 | FTR | 42 | 30 | 28 | FTR | FTR | FTR | Clay loam (USDA)** | FTR | FTR | 1.6 | FTR | 1.6 | FTR | FTR | 10 (EST*) | 2–7 (EST*) | Clay loam — modelled texture estimate (Sand 4… | NO DATA (GIS) |

### Table 2 — Aggregated intervals 0–1 m and 1–2 m

| Depth (m) | Gravel % | Sand % | Silt % | Clay % | LL (%) | PL (%) | PI (%) | Soil Class | MDD (g/cc) | OMC (%) | Dry Density (g/cc) | FSI (%) | Bulk Density (g/cc) | UCS (kg/cm²) | SG | SBC (T/m²) | CBR (%) | Remarks | Ground Water Table (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0–1 | FTR | 36.9 | 30.2 | 32.9 | FTR | FTR | FTR | Clay loam (USDA)** | FTR | FTR | 1.5 | FTR | 1.5 | FTR | FTR | 10 (EST*) | 2–7 (EST*) | Clay loam — modelled texture estimate (Sand 3… | NO DATA (GIS) |
| 1–2 | FTR | 42 | 30 | 28 | FTR | FTR | FTR | Clay loam (USDA)** | FTR | FTR | 1.6 | FTR | 1.6 | FTR | FTR | 10 (EST*) | 2–7 (EST*) | Clay loam — modelled texture estimate (Sand 4… | NO DATA (GIS) |

• FTR = FIELD TEST REQUIRED (laboratory / in-situ). Values are never fabricated as MEASURED.

• Sand / Silt / Clay / Dry Density / Bulk Density: MODELLED from ISRIC SoilGrids 2.0 (thickness-weighted).

• Soil Class marked (USDA)** is texture-triangle class — not IS 1498 without Atterberg limits.

• SBC/CBR marked (EST*) = texture screening estimate — not IS 6403 CALCULATED / not soaked CBR lab result.

• SBC marked (CALC) = IS 6403 net safe bearing capacity when c, φ, γ inputs are scientifically complete.

• Gravel % requires sieve analysis; SoilGrids cfvo is not reported as gravel %.

# 2. ENGINEERING ANALYSIS

## 2.1 Foundation Design Parameters

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| Foundation Type | 1.0 m × 1.0 m square isolated stub footing (screening assumption) | IS 6403 : 1981 |
| Depth of Foundation (Df) | 1.5 m | Site / screening condition |
| Allowable Settlement | 25 mm | IS 8009 (Part-1) |
| Factor of Safety (shear) | 2.5 | IS 6403 |
| Water Table | NO DATA (GIS) — not invented | Preliminary assessment |
| Width B / Length L | 1 m × 1 m | Screening default |

## 2.2 Adopted Soil Parameters (at 1.5 m depth — screening)

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| Unit weight γ | 15.7 | ESTIMATED |
| Friction angle φ | 22 | ESTIMATED |
| Cohesion c | FIELD TEST REQUIRED | FIELD TEST REQUIRED |
| Texture hint | Clay loam | MODELLED / ESTIMATED |
| Code reference | IS 6403:1981 — Code of practice for determination of bearing capacity of shallow founda… |  |

Note: Adopted c, φ, γ for GIS screening are ESTIMATED or FIELD TEST REQUIRED. They are not borehole MEASURED values unless a matched field investigation is identified.

## 2.4 SBC CALCULATION — DETAILED SAMPLE

Cannot compute IS 6403 SBC — missing φ, γ, and/or cohesion (clayey soils need lab shear; drained c≈0 only for sand-like textures).

Calculation status: INSUFFICIENT_DATA.

Safe bearing capacity (IS 6403) could not be completed because cohesion (for clayey soils), friction angle, and/or unit weight are unavailable or outside validity. Texture-based indicative SBC shown in Soil Test Summary as (EST*) must not be treated as CALCULATED net safe bearing capacity.

Step 1 — Input validation: required: c (or drained sand assumption), φ, γ, B, L, Df, FoS (Cohesion c unavailable — clayey soils require lab shear tests; drained c=0 only applied for sand-like textures)

## 2.5 SBC Values for Location

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| Location | Patna, Bihar | 25.594100°N 85.137600°E |
| φ (°) | 22 | ESTIMATED |
| c | FIELD TEST REQUIRED | FIELD TEST REQUIRED |
| γ | 15.7 | ESTIMATED |
| SBC (T/m²) | INSUFFICIENT DATA | INSUFFICIENT_DATA |

## 2.6 SBC Variation with Depth

| Depth (m) | Net Safe Bearing Capacity (T/m²) | Status |
| --- | --- | --- |
| 0.5 | INSUFFICIENT DATA | INSUFFICIENT DATA |
| 1.0 | INSUFFICIENT DATA | INSUFFICIENT DATA |
| 1.5 | INSUFFICIENT DATA | INSUFFICIENT DATA |
| 2.0 | INSUFFICIENT DATA | INSUFFICIENT DATA |

## 2.7 Settlement Check

Final settlement not computed — tower foundation load / contact pressure required from structural design

Calculation status: TOWER_LOAD_REQUIRED.

Required inputs: TOWER_FOUNDATION_LOAD, FOUNDATION_DIMENSIONS, SOIL_MODULUS_OR_EQUIVALENT, POISSON_RATIO, INFLUENCE_FACTOR.

Missing inputs: TOWER_FOUNDATION_LOAD, SOIL_MODULUS_OR_EQUIVALENT.

Final settlement value is not reported. Tower foundation load / contact pressure and/or soil modulus are required before a numerical settlement can be stated.

## 2.8 Final Recommended SBC for Design

No CALCULATED IS 6403 net SBC is adopted for design from GIS-only inputs. Use Soil Test Summary (EST*) values only for planning / screening, and commission field investigation (SPT / shear / Atterberg) before foundation design.

# 3. PILE FOUNDATION ANALYSIS & DESIGN

## 3.1 Design Parameters

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| Pile Type | Cast-in-situ RCC pile (screening) | IS 2911 (Part 1) |
| Pile Diameters | 450 mm and 600 mm | TAMS tower screening |
| Pile Depths | 1.0 m, 1.5 m, 2.0 m |  |
| FoS Compression | 2.5 | IS 2911 practice |
| FoS Uplift | 3.0 | IS 2911 practice |
| Method | c–φ static method only; SPT N never fabricated | IS 2911 (Part 1/Sec 2):2010 — bored cast-in-situ piles (static screening form) |

SPT N-values are never fabricated. Lateral capacity requires tower shear/moment and pile-head fixity.

Pile capacities INSUFFICIENT_DATA for all configurations — need φ, γ, and c (or drained-sand c′≈0). SPT N is unavailable and will not be fabricated. (6/6 blocked)

## 3.2 Adopted Soil Parameters (Layer-wise)

| Layer | Depth (m) | Soil Type | c | φ (°) | γ |
| --- | --- | --- | --- | --- | --- |
| Layer I | 0.0–0.5 | Clay loam | FIELD TEST REQUIRED | 22 | 15.7 |
| Layer II | 0.5–1.0 | Clay loam | FIELD TEST REQUIRED | 22 | 15.7 |
| Layer III | 1.0–1.5 | Clay loam | FIELD TEST REQUIRED | 22 | 15.7 |
| Layer IV | 1.5–2.0 | Clay loam | FIELD TEST REQUIRED | 22 | 15.7 |

## 3.3 Cross-Sectional Area of Piles

450 mm — Ap = πD²/4                 =  0.159 m²

600 mm — Ap = πD²/4                 =  0.2827 m²

## 3.4 VERTICAL LOAD CAPACITY (Detailed Sample — 450 mm × 2.0 m)

Diameter D                          =  0.450 m

Length L                            =  2.000 m

Calculation status                  =  INSUFFICIENT_DATA

Area Ap                             =  0.159 m²

Perimeter                           =  1.4137 m

φ                                   =  22°

c                                   =  INSUFFICIENT DATA

γ                                   =  1.6 T/m³

Calculation could not be completed because the following field parameters are unavailable: LAYER_COHESION_c.

## 3.5 Summary — Vertical Capacity

| Diameter | Depth | Safe Vertical (T) | Status |
| --- | --- | --- | --- |
| 450 mm | 1.0 m | INSUFFICIENT DATA | INSUFFICIENT DATA |
| 450 mm | 1.5 m | INSUFFICIENT DATA | INSUFFICIENT DATA |
| 450 mm | 2.0 m | INSUFFICIENT DATA | INSUFFICIENT DATA |
| 600 mm | 1.0 m | INSUFFICIENT DATA | INSUFFICIENT DATA |
| 600 mm | 1.5 m | INSUFFICIENT DATA | INSUFFICIENT DATA |
| 600 mm | 2.0 m | INSUFFICIENT DATA | INSUFFICIENT DATA |

## 3.6 UPLIFT CAPACITY (Sample — 450 mm × 2.0 m)

Safe uplift: INSUFFICIENT DATA (INSUFFICIENT DATA). When CALCULATED, uplift uses shaft resistance / FoS uplift per IS 2911 screening form.

## 3.7 UPLIFT LOAD CAPACITY SUMMARY

| Diameter | Depth | Safe Uplift (T) | Status |
| --- | --- | --- | --- |
| 450 mm | 1.0 m | INSUFFICIENT DATA | INSUFFICIENT DATA |
| 450 mm | 1.5 m | INSUFFICIENT DATA | INSUFFICIENT DATA |
| 450 mm | 2.0 m | INSUFFICIENT DATA | INSUFFICIENT DATA |
| 600 mm | 1.0 m | INSUFFICIENT DATA | INSUFFICIENT DATA |
| 600 mm | 1.5 m | INSUFFICIENT DATA | INSUFFICIENT DATA |
| 600 mm | 2.0 m | INSUFFICIENT DATA | INSUFFICIENT DATA |

## 3.8 LATERAL LOAD CAPACITY

| Diameter | Depth | Lateral Capacity (T) | Status |
| --- | --- | --- | --- |
| 450 mm | 1.0 m | FIELD TEST REQUIRED | FIELD TEST REQUIRED |
| 450 mm | 1.5 m | FIELD TEST REQUIRED | FIELD TEST REQUIRED |
| 450 mm | 2.0 m | FIELD TEST REQUIRED | FIELD TEST REQUIRED |
| 600 mm | 1.0 m | FIELD TEST REQUIRED | FIELD TEST REQUIRED |
| 600 mm | 1.5 m | FIELD TEST REQUIRED | FIELD TEST REQUIRED |
| 600 mm | 2.0 m | FIELD TEST REQUIRED | FIELD TEST REQUIRED |

## 3.9 Governing Design Condition for Transmission Tower Foundation

Transmission line tower foundations are generally governed by vertical compression, uplift due to wind/conductor imbalance, and lateral wind loads (IS 875 Part 3). GIS screening does not replace structural load combinations or field pile design.

## 3.10 Detailed Configuration Sheets (all diameters × depths)

### Pile Foundation — 450 mm Diameter

### 450 mm dia × 1.0 m depth

Diameter D                          =  0.450 m

Length L                            =  1.000 m

Calculation status                  =  INSUFFICIENT_DATA

Area Ap                             =  0.159 m²

Perimeter                           =  1.4137 m

φ                                   =  22°

c                                   =  INSUFFICIENT DATA

γ                                   =  1.6 T/m³

Calculation could not be completed because the following field parameters are unavailable: LAYER_COHESION_c.

| Capacity | Result | Status |
| --- | --- | --- |
| Safe vertical | INSUFFICIENT DATA | INSUFFICIENT DATA |
| Safe uplift | INSUFFICIENT DATA | INSUFFICIENT DATA |
| Lateral | FIELD TEST REQUIRED | FIELD TEST REQUIRED |

### 450 mm dia × 1.5 m depth

Diameter D                          =  0.450 m

Length L                            =  1.500 m

Calculation status                  =  INSUFFICIENT_DATA

Area Ap                             =  0.159 m²

Perimeter                           =  1.4137 m

φ                                   =  22°

c                                   =  INSUFFICIENT DATA

γ                                   =  1.6 T/m³

Calculation could not be completed because the following field parameters are unavailable: LAYER_COHESION_c.

| Capacity | Result | Status |
| --- | --- | --- |
| Safe vertical | INSUFFICIENT DATA | INSUFFICIENT DATA |
| Safe uplift | INSUFFICIENT DATA | INSUFFICIENT DATA |
| Lateral | FIELD TEST REQUIRED | FIELD TEST REQUIRED |

### 450 mm dia × 2.0 m depth

Diameter D                          =  0.450 m

Length L                            =  2.000 m

Calculation status                  =  INSUFFICIENT_DATA

Area Ap                             =  0.159 m²

Perimeter                           =  1.4137 m

φ                                   =  22°

c                                   =  INSUFFICIENT DATA

γ                                   =  1.6 T/m³

Calculation could not be completed because the following field parameters are unavailable: LAYER_COHESION_c.

| Capacity | Result | Status |
| --- | --- | --- |
| Safe vertical | INSUFFICIENT DATA | INSUFFICIENT DATA |
| Safe uplift | INSUFFICIENT DATA | INSUFFICIENT DATA |
| Lateral | FIELD TEST REQUIRED | FIELD TEST REQUIRED |

### Pile Foundation — 600 mm Diameter

### 600 mm dia × 1.0 m depth

Diameter D                          =  0.600 m

Length L                            =  1.000 m

Calculation status                  =  INSUFFICIENT_DATA

Area Ap                             =  0.2827 m²

Perimeter                           =  1.885 m

φ                                   =  22°

c                                   =  INSUFFICIENT DATA

γ                                   =  1.6 T/m³

Calculation could not be completed because the following field parameters are unavailable: LAYER_COHESION_c.

| Capacity | Result | Status |
| --- | --- | --- |
| Safe vertical | INSUFFICIENT DATA | INSUFFICIENT DATA |
| Safe uplift | INSUFFICIENT DATA | INSUFFICIENT DATA |
| Lateral | FIELD TEST REQUIRED | FIELD TEST REQUIRED |

### 600 mm dia × 1.5 m depth

Diameter D                          =  0.600 m

Length L                            =  1.500 m

Calculation status                  =  INSUFFICIENT_DATA

Area Ap                             =  0.2827 m²

Perimeter                           =  1.885 m

φ                                   =  22°

c                                   =  INSUFFICIENT DATA

γ                                   =  1.6 T/m³

Calculation could not be completed because the following field parameters are unavailable: LAYER_COHESION_c.

| Capacity | Result | Status |
| --- | --- | --- |
| Safe vertical | INSUFFICIENT DATA | INSUFFICIENT DATA |
| Safe uplift | INSUFFICIENT DATA | INSUFFICIENT DATA |
| Lateral | FIELD TEST REQUIRED | FIELD TEST REQUIRED |

### 600 mm dia × 2.0 m depth

Diameter D                          =  0.600 m

Length L                            =  2.000 m

Calculation status                  =  INSUFFICIENT_DATA

Area Ap                             =  0.2827 m²

Perimeter                           =  1.885 m

φ                                   =  22°

c                                   =  INSUFFICIENT DATA

γ                                   =  1.6 T/m³

Calculation could not be completed because the following field parameters are unavailable: LAYER_COHESION_c.

| Capacity | Result | Status |
| --- | --- | --- |
| Safe vertical | INSUFFICIENT DATA | INSUFFICIENT DATA |
| Safe uplift | INSUFFICIENT DATA | INSUFFICIENT DATA |
| Lateral | FIELD TEST REQUIRED | FIELD TEST REQUIRED |

# 4. CALIFORNIA BEARING RATIO (CBR)

Measured (soaked) CBR and estimated CBR are reported separately. Estimated values must not be labelled as laboratory soaked CBR.

| Depth interval | Measured CBR (%) | Estimated CBR (%) | Measured status | Estimated status |
| --- | --- | --- | --- | --- |
| 0.0–0.5 m | NO DATA | 2–7 | NO DATA | ESTIMATED |
| 0.5–1.0 m | NO DATA | 2–7 | NO DATA | ESTIMATED |
| 1.0–1.5 m | NO DATA | 2–7 | NO DATA | ESTIMATED |
| 1.5–2.0 m | NO DATA | 2–7 | NO DATA | ESTIMATED |

# 5. EARTH RESISTIVITY

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| Measured earth resistivity | NO DATA | NO DATA |
| Estimated soil resistivity | NO DATA | NO DATA |

Estimated resistivity is never presented as a Wenner field measurement. Where no defensible correlation inputs exist, status remains NO DATA.

# 6. Foundation Recommendation

Based on the available gis based preliminary screening, the following preliminary observations are recorded for engineering guidance only:

1. Subsurface screening to 2.0 m indicates modelled soil conditions as summarised in Sections 4–5. Laboratory confirmation is required before final foundation design.

2. Net safe bearing capacity: not calculated — INSUFFICIENT DATA for IS 6403 inputs.

3. Pile foundations (450 mm / 600 mm): vertical and uplift capacities are reported only where c–φ static inputs exist; otherwise FIELD TEST REQUIRED / INSUFFICIENT DATA. Lateral capacity remains blocked pending structural loads.

4. Recommended next field works: SPT or shear testing, Atterberg limits, soaked CBR, groundwater observation, and Wenner resistivity as listed in Section 7.

These recommendations do not constitute a sealed structural design certificate.

# 7. Investigation Limitations

1. PRELIMINARY GIS-BASED GEOTECHNICAL SCREENING — not a substitute for borehole investigation.

2. SoilGrids values are modelled at ~250 m resolution; report intervals are aggregated from source depth bands.

3. bdod is bulk density (g/cm³) and must never be interpreted as soil depth.

4. NO_DATA parameters are null — never replaced with 0.

5. SPT N, Atterberg limits, soaked CBR, groundwater depth, and field resistivity require physical testing.

6. Preliminary φ (if present) is ESTIMATED from texture mid-ranges; cohesion is FIELD_TEST_REQUIRED except drained sand c′≈0 ESTIMATED.

7. SBC: INSUFFICIENT_DATA — IS 6403 not run without valid φ, γ, and c (or drained-sand assumption).

8. Settlement: TOWER_LOAD_REQUIRED — Final settlement not computed — tower foundation load / contact pressure required from structural design

9. Pile: Pile capacities INSUFFICIENT_DATA for all configurations — need φ, γ, and c (or drained-sand c′≈0). SPT N is unavailable and will not be fabricated. (6/6 blocked)

10. SoilGrids texture class (0–30 cm screening): Clay loam

11. No field investigation within search radius

Missing critical data: SPT_N_VALUE; DIRECT_SHEAR_PARAMETERS; GROUNDWATER_LEVEL; FIELD_EARTH_RESISTIVITY; ATTERBERG_LIMITS; SOAKED_CBR.

# 8. References

1. IS 6403:1981 — Code of practice for determination of bearing capacity of shallow foundations.

2. IS 2911 (Part 1/Sec 2):2010 — Design and construction of pile foundations — Bored cast-in-situ piles.

3. IS 1498 — Classification and identification of soils for general engineering purposes.

4. IS 2720 (relevant parts) — Methods of test for soils.

5. IS 3043 — Code of practice for earthing.

6. ISRIC SoilGrids 2.0 — Global gridded soil information (modelled properties).

7. TAMS Tower Suitability — Geotechnical Intelligence module (GEO-1) calculation sheets.

# ANNEXURE A — Depth-wise Soil Data

Engineering depth intervals used in the main report (metres only). Sand / silt / clay / density values below are MODELLED SoilGrids means — not laboratory sieve or Proctor results.

| Report depth | Sand % | Silt % | Clay % | Bulk dens. | Dry dens. | pH | Org. C | USDA | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.0–0.5 m | 34.6 | 30.6 | 34.8 | 1.45 | 1.45 | 7 | 7 | Clay loam | MODELLED |
| 0.5–1.0 m | 39.2 | 29.8 | 31 | 1.54 | 1.54 | 6.82 | 3.4 | Clay loam | MODELLED |
| 1.0–1.5 m | 42 | 30 | 28 | 1.6 | 1.6 | 6.7 | 2 | Clay loam | MODELLED |
| 1.5–2.0 m | 42 | 30 | 28 | 1.6 | 1.6 | 6.7 | 2 | Clay loam | MODELLED |

# ANNEXURE B — Source Data and Provenance

The following centimetre-scale intervals are SoilGrids source dataset bands. They are not laboratory sample depths and must not be confused with the engineering metre intervals in the main report.

| Source depth | From (m) | To (m) | Sand % | Silt % | Clay % | bdod (g/cm³) | pH | cfvo % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0-5cm | 0.00 | 0.05 | 35 | 33 | 32 | 1.35 | 7.2 | 4 |
| 5-15cm | 0.05 | 0.15 | 34 | 32 | 34 | 1.4 | 7.1 | 5 |
| 15-30cm | 0.15 | 0.30 | 33 | 31 | 36 | 1.45 | 7 | 5.5 |
| 30-60cm | 0.30 | 0.60 | 36 | 29 | 35 | 1.5 | 6.9 | 6 |
| 60-100cm | 0.60 | 1.00 | 40 | 30 | 30 | 1.55 | 6.8 | 7 |
| 100-200cm | 1.00 | 2.00 | 42 | 30 | 28 | 1.6 | 6.7 | 8 |

Aggregation method for report intervals: thickness-weighted mean of overlapping SoilGrids source layers.

Field investigation match: No field investigation within search radius

# ANNEXURE C — Engineering Calculation Sheets

This annexure consolidates calculation status for SBC, settlement and piles. Detailed step lists appear in Sections 6–7 when CALCULATED or PARTIAL.

| Parameter | Value / Status | Remarks |
| --- | --- | --- |
| SBC status | INSUFFICIENT_DATA | IS 6403:1981 — Code of practice for determination of bearing capacity of shallow founda… |
| Settlement status | TOWER_LOAD_REQUIRED | Elastic settlement screening (form consistent with IS 8009 elastic approach) — prelimin… |
| Pile 450 mm × 2.0 m | INSUFFICIENT_DATA | INSUFFICIENT DATA |
| Pile 600 mm × 2.0 m | INSUFFICIENT_DATA | INSUFFICIENT DATA |

— End of Report —