# TAMS Tower Suitability — Factor Merge Reference

What is merged during Analyze · How 12 scoring factors combine · TAMS open-data screening only

## 1. Final merge — all 12 factors → one score

When you click Analyze, each factor is scored 0–10, then merged into a single site score.

| Formula | Detail |
| --- | --- |
| Site score (0–10) | Σ (factor_score × weight) ÷ Σ weights |
| Verdict | ≥ 7.0 Preferred · 4.5–7.0 Conditional · < 4.5 Unsuitable |

### All 12 factors in the merge

| # | Factor | Weight | Merge role |
| --- | --- | --- | --- |
| 1 | Terrain slope | 22% | Shares DEM fetch with #8 |
| 2 | Road access | 14% | Standalone (OSRM) |
| 3 | Water / flood buffer | 14% | Feeds #12 corridor check |
| 4 | Settlement clearance | 10% | Feeds #12 corridor check |
| 5 | Power connectivity | 10% | Shares TAMS+OSM with #6, #7 |
| 6 | Voltage suitability | 8% | Shares TAMS+OSM with #5, #7 |
| 7 | Connection distance | 8% | Shares TAMS+OSM with #5, #6 |
| 8 | Elevation | 8% | Shares DEM fetch with #1 |
| 9 | Land cover | 8% | Standalone (OSM + Nominatim) |
| 10 | Soil / SBC | 8% | TAMS geotech OR SoilGrids → one factor |
| 11 | Wind | 6% | Standalone (Open-Meteo) |
| 12 | Corridor feasibility | 6% | Merges #1 + #3 + #4 signals |

## 2. Data merges before scoring (collectSiteSignals)

During Analyze, live APIs are fetched in parallel, merged into shared signal buckets, then passed to scoreSiteSignals().

### A. DEM → 2 factors (#1 Slope + #8 Elevation)

| Source | What is merged | Feeds factors |
| --- | --- | --- |
| Open-Meteo Elevation API | 5-point grid: centre + N/S/E/W (~133 m). Centre → elevation. Max angle → slope. | #1 Terrain slope · #8 Elevation |

### B. TAMS GIS + OSM Overpass → 3 factors (#5, #6, #7)

| Source | Assets merged | Feeds factors |
| --- | --- | --- |
| TAMS GIS | Towers, substations, transmission lines | #5 Power connectivity #6 Voltage suitability #7 Connection distance |
| OSM Overpass | Towers, portals, poles (≤15 km), lines, cables, substations, plants | (same merge list) |

• #5 — nearest asset distance + bonuses (pole ≤350 m, tower ≤2 km, substation, interconnect ease)

• #6 — voltage tags from merged assets vs planning tiers (11–765 kV)

• #7 — practical spur = nearest distance × 1.2

### C. TAMS Geotech OR SoilGrids → 1 factor (#10)

| Priority | Source | Rule |
| --- | --- | --- |
| 1 (preferred) | TAMS field geotech within ~5 km | Uses adopted SBC, CBR, design depth from /geotech |
| 2 (fallback) | ISRIC SoilGrids 2.0 | Texture class, indicative SBC/CBR, ~40–48% screening confidence |

Only one path is used — field geotech wins when present; otherwise open GIS soil screening.

### D. Water + Settlement + Slope → 1 factor (#12)

Factor #12 does not fetch new data. It reuses signals already computed for other factors:

| Reused signal | From factor | Penalty if triggered |
| --- | --- | --- |
| Water < 150 m | #3 Water / flood buffer | −2 |
| Settlement < 100 m | #4 Settlement clearance | −2 |
| Slope > 12° | #1 Terrain slope | −1.5 |
| Slope > 18° | #1 Terrain slope | −1 |

Starts at score 8; deductions applied; capped 0–10.

## 3. Standalone factors (no merge with others)

| # | Factor | Source only |
| --- | --- | --- |
| 2 | Road access | OSRM nearest drivable road |
| 3 | Water / flood buffer | OSM water features (+ Photon fallback if Overpass fails) |
| 4 | Settlement clearance | OSM buildings/places (+ Photon fallback) |
| 9 | Land cover | OSM landuse/natural + Nominatim fallback |
| 11 | Wind exposure | Open-Meteo 90-day mean daily max wind @ 10 m |

## 4. Confidence % — partial merge (not all 12)

Confidence counts 7 resolved signals: elevation, slope, road, water, settlement, grid (tower/SS), wind.

Not in confidence directly: #6 Voltage · #7 Connection distance · #9 Land cover · #10 Soil · #12 Corridor feasibility

Formula: base 55% + up to 25% (resolved ÷ 7 × 25) − 5% per Photon/fallback used.

## 5. NOT merged into the 12-factor score

| Module | Runs at analyze? | In /10 score? |
| --- | --- | --- |
| Geotechnical Intelligence (GEO-1 Word/HTML report) | Yes | No — outside scoreSiteSignals |
| Corridor placement panel (Suggest/Shift/Skip/Review) | Yes (KML/line) | No — separate CEA span screening |
| Power network verdict (Yes/No/Unknown) | Yes | No — informational only |
| OSRM road route overlay (orange line) | On demand | No — display only |

## 6. Quick reference — merge groups

| Merge group | Factors affected | Shared data |
| --- | --- | --- |
| DEM group | #1, #8 | Open-Meteo 5-point elevation grid |
| Power group | #5, #6, #7 | TAMS + OSM deduped nearbyPower |
| Soil group | #10 | TAMS geotech OR SoilGrids (one path) |
| Corridor group | #12 (uses #1, #3, #4) | Water km + building km + slope ° |
| Final score | All #1–#12 | Weighted average → verdict |

This document describes how TAMS Tower Suitability merges open-data signals during Analyze. Screening logic only — not utility approval, ROW certification, or foundation design.