# TAMS Tower Suitability — Numbered Analysis Reference

What you see · What we fetch · What we calculate · Factors 1–12 with sub-points

## 0. When you click Analyze — overall flow

| Step | What you see | What we calculate |
| --- | --- | --- |
| 0.1 | Progress bar: DEM, OSM, roads, weather, soil | Parallel API fetch at focus point |
| 0.2 | Map: pad, road snap, grid assets, shift ghosts | Distances, OSRM, TAMS+OSM merge |
| 0.3 | Score card: X.X/10 · verdict · confidence % | Weighted average of 12 factors |
| 0.4 | Factors panel: 12 bars + values | Each factor 0–10 |
| 0.5 | Breakdown table: Value·Score·Weight·Contrib | Contrib = score × weight |
| 0.6 | GeoTech tab: Word report | Separate — NOT in /10 |
| 0.7 | Corridor: Suggest/Shift/Skip/Review | CEA spans — NOT in /10 |

## 1. Terrain slope (Weight 22%)

### 1.1 What you see

• Factor bar labelled Terrain slope with score 0–10

• Value line: e.g. 8.4° or n/a

• Note if steep: pad grading / retention likely costly

• Source tag: Live · Open-Meteo DEM

### 1.2 What we fetch

• Open-Meteo Elevation API → Copernicus DEM ~30 m

• 5 sample points: centre + N/S/E/W at 0.0012° (~133 m)

### 1.3 What we calculate

• For each offset: run distance (m), rise = |elev offset − centre|

• Angle = atan(rise/run) × 180/π

• Output slope = maximum angle among 4 directions

### 1.4 Score calculation

| Condition | Score |
| --- | --- |
| ≤ 5° | 10 (best) |
| ≥ 18° | 0 (worst) |
| Between | Linear interpolation |
| Missing | 5 neutral |

### 1.5 Contribution to final score

Contrib = factor score × 0.22

## 2. Road access (Weight 14%)

### 2.1 What you see

• Factor bar: Road access

• Value: e.g. 1.24 km or live lookup failed

• Map: orange marker at nearest road snap

• Source: Live · OSRM nearest road

### 2.2 What we fetch

• OSRM Nearest API — snap pad to drivable OSM road

### 2.3 What we calculate

• Road distance (km) = OSRM waypoint distance ÷ 1000

• Fallback: Haversine pad → snap if OSRM distance missing

### 2.4 Score calculation

| Condition | Score |
| --- | --- |
| ≤ 0.5 km | 10 |
| ≥ 8 km | 0 |
| Lookup failed | 3 uncertain |

### 2.5 Contribution to final score

Contrib = factor score × 0.14

## 3. Water / flood buffer (Weight 14%)

### 3.1 What you see

• Factor bar: Water / flood buffer

• Value: e.g. 2.15 km, > 8 km, or fallback warning

• Source: Live · OSM Overpass or Fallback · Photon

### 3.2 What we fetch

• OSM Overpass 8 km: water, waterway, reservoir, basin

• Fallback: Photon lake/river/reservoir — confidence −5%

### 3.3 What we calculate

• Nearest water distance (km) via Haversine

• Cap at 8 km for scoring

### 3.4 Score calculation

| Condition | Score |
| --- | --- |
| ≥ 0.8 km | 10 far = good |
| ≤ 0.05 km | 0 flood risk |
| Missing | 5 neutral |

### 3.5 Contribution to final score

Contrib = score × 0.14 · Reused in #12

## 4. Settlement clearance (Weight 10%)

### 4.1 What you see

• Factor bar + value e.g. 1.80 km or > 4 km

• Note if close to buildings

### 4.2 What we fetch

• OSM Overpass 4 km: buildings, places, residential

• Photon fallback if Overpass fails

### 4.3 What we calculate

• Nearest settlement distance (km)

• Cap at 4 km for scoring

### 4.4 Score calculation

| Condition | Score |
| --- | --- |
| ≥ 0.25 km | 10 |
| ≤ 0.02 km | 0 |
| Missing | 6 |

### 4.5 Contribution to final score

Contrib = score × 0.10 · Reused in #12

## 5. Power connectivity (Weight 10%)

### 5.1 What you see

• Value: Tower/Line name · distance km

• Power panel: nearest assets list

• Map: grid markers, connection lines

### 5.2 What we fetch

• TAMS GIS: towers, SS, lines

• OSM: tower, portal, pole, line, cable, SS, plant

• Deduped into nearbyPower list

### 5.3 What we calculate

• Haversine distances; pick nearest asset

• Interconnect ease: ≤0.25/2/12 km thresholds

### 5.4 Score calculation

| Condition | Score |
| --- | --- |
| Base | 0.2 km→10, 20 km→0 linear |
| Bonuses | Pole +2, tower +1.2, SS +0.8, easy +0.5 |
| No data | 5 neutral |

### 5.5 Contribution to final score

Contrib = score × 0.10 · Shares data with #6, #7

## 6. Voltage suitability (Weight 8%)

### 6.1 What you see

• Value: e.g. 220 kV, 33/110 kV, unknown

• Controls: selected line class kV

### 6.2 What we fetch

• Voltage tags from merged power list (#5)

• Tiers: 11–765 kV

### 6.3 What we calculate

• Available kV set

• Suggested kV from corridor → tower → line → SS

### 6.4 Score calculation

| Condition | Score |
| --- | --- |
| Tagged + HV ≥33 kV | ~9 |
| Tagged present | ~7 |
| No tags | 5 neutral |

### 6.5 Contribution to final score

Contrib = score × 0.08

## 7. Connection distance (Weight 8%)

### 7.1 What you see

• Value: ~480 m or ~3.2 km

• Asset card: straight m + road route m

### 7.2 What we fetch

• Nearest asset from power merge (#5)

### 7.3 What we calculate

• Direct Haversine (km)

• Practical spur = direct × 1.2

### 7.4 Score calculation

| Condition | Score |
| --- | --- |
| ≤ 0.3 km | 10 |
| ≥ 15 km | 0 |
| Unavailable | 5 |

### 7.5 Contribution to final score

Contrib = score × 0.08

## 8. Elevation (Weight 8%)

### 8.1 What you see

• Value: e.g. 342 m AMSL

• Same DEM as #1

### 8.2 What we fetch

• Centre elevation from Open-Meteo 5-point grid

### 8.3 What we calculate

• Elevation at pad centre (m)

### 8.4 Score calculation

| Condition | Score |
| --- | --- |
| 20–800 m | 9 |
| < 5 m | 3 |
| > 1800 m | 4 |
| Other/missing | 7 |

### 8.5 Contribution to final score

Contrib = score × 0.08

## 9. Land cover hint (Weight 8%)

### 9.1 What you see

• Value: Barren / Vegetated / Built-up / Water / Unknown

### 9.2 What we fetch

• OSM landuse/natural 180 m

• Nominatim fallback

### 9.3 What we calculate

• 5-class keyword classification

### 9.4 Score calculation

| Condition | Score |
| --- | --- |
| Barren | 9 |
| Vegetated | 5 |
| Built-up | 2 |
| Water | 0 |
| Unknown | 6 |

### 9.5 Contribution to final score

Contrib = score × 0.08

## 10. Soil / SBC (Weight 8%)

### 10.1 What you see

• Value: Loam · ~45% or GT-001 · 2.1 km

• GeoTech tab: Word report, soil tables

### 10.2 What we fetch

• Path A: TAMS geotech ≤5 km

• Path B: ISRIC SoilGrids 2.0

### 10.3 What we calculate

• Path A: score from adopted SBC, CBR

• Path B: texture, indicative SBC, confidence%

• GeoTech report calcs outside /10 score

### 10.4 Score calculation

| Condition | Score |
| --- | --- |
| Field SBC ≥20 | 9 |
| Field SBC ≥12 | 7 |
| SoilGrids only | max(5, conf%÷10) |
| Neither | 5 |

### 10.5 Contribution to final score

Contrib = score × 0.08

## 11. Wind exposure (Weight 6%)

### 11.1 What you see

• Value: e.g. 6.2 m/s

### 11.2 What we fetch

• Open-Meteo 90-day daily max wind @ 10 m

### 11.3 What we calculate

• Mean of daily max wind (m/s)

### 11.4 Score calculation

| Condition | Score |
| --- | --- |
| ≤ 4 m/s | 10 |
| ≥ 12 m/s | 0 |
| Missing | 6 |

### 11.5 Contribution to final score

Contrib = score × 0.06

## 12. Corridor feasibility (Weight 6%)

### 12.1 What you see

• Value: obstruction list or No major obstruction

• Reuses #1, #3, #4 — no new fetch

### 12.2 What we fetch

• Reuses water km, building km, slope°

### 12.3 What we calculate

• Start at 8, apply penalties, clamp 0–10

### 12.4 Score calculation

| Condition | Score |
| --- | --- |
| Water <150 m | −2 |
| Settlement <100 m | −2 |
| Slope >12° | −1.5 |
| Slope >18° | −1 |

### 12.5 Contribution to final score

Contrib = score × 0.06

## 13. Final score merge (all 12 factors)

### 13.1 What you see

• Header X.X/10

• Verdict: Preferred / Conditional / Unsuitable

• Confidence %

### 13.2 What we calculate

| Output | Formula |
| --- | --- |
| Final score | Σ(factor_score × weight) ÷ Σ weights |
| Verdict | ≥7 Preferred · 4.5–7 Conditional · <4.5 Unsuitable |

## 14. Confidence % (7 signals, not all 12)

• Counts: elevation, slope, road, water, settlement, grid, wind

• Base 55% + (resolved÷7)×25% − 5% per fallback

• NOT counted: voltage, connection, land cover, soil, corridor

## 15. Separate from /10 score

| # | Module | What you see | What we calculate |
| --- | --- | --- | --- |
| 15.1 | GeoTech Word report | Soil tables, SBC/pile download | IS 6403/2911 — outside score |
| 15.2 | Corridor T1…Tn | Suggest/Shift/Skip/Review | CEA span bands |
| 15.3 | Power take-off | Best pad → SS | Haversine + ease |
| 15.4 | Road route overlay | Orange polyline | OSRM display only |
| 15.5 | Shift ghosts | Suggested offset pads | Spacing shift rules |

## 16. Merge groups — quick lookup

| Group | Factors | Shared fetch |
| --- | --- | --- |
| DEM | #1 + #8 | Open-Meteo 5-point elevation |
| Power | #5 + #6 + #7 | TAMS + OSM nearbyPower |
| Soil | #10 | TAMS geotech OR SoilGrids |
| Corridor | #12 uses #1+#3+#4 | Reused distances/slope |
| Final | All #1–#12 | Weighted average |

Open-data screening only. Not utility approval or foundation design.